# Work 6. H-infinity optimal control design

# Problem statement

Disturbed linear plant of variant 5:

x˙=Ax+Bu+Bff,x(0)=[1, 0]T

A=62−30, B=03, Bf=41, Q=diag(2,7)

f=10sin6t+5cos2t+4cos3t+3cos8t

# Synthesis and minimal gamma

The H-infinity Riccati equation is solved while decreasing gamma until a stabilizing positive semi-definite P no longer exists; that boundary gives gamma_min. A coarse grid locates the boundary and a bisection then refines it to the tolerance below. Exactly at gamma_min the solution P is unbounded, so the controller u=Kx with K=-B'P is synthesized at a slightly suboptimal gamma = 1.01 gamma_min, which keeps P bounded and the closed loop realizable; u and the states are plotted for this gamma.

ATP+PA+Q−PBBTP+γ−2PBfBfTP=0

K=−BTP

gamma_min = 9.4549 (bisection tolerance 0.0001)

design gamma = 9.5494

K = ['-1407.2370', '-208.3269']

closed-loop eig real parts = ['-7.6879', '-611.2929']

# H-infinity norms of disturbance transfer functions

Norms are computed by a frequency sweep of the closed-loop transfer function from the disturbance to each output.

||C1 (sI-Acl)^-1 Bf||_inf = 0.5324

||C2 (sI-Acl)^-1 Bf||_inf = 3.5971

||(sI-Acl)^-1 Bf||_inf = 3.6363