# Work 3. LQR controller design in Simulink (variant 5)

# Problem statement

For the linear time-invariant plant of variant 5 the goal is to design the optimal state-feedback controller u = -Kx that minimises the quadratic cost over an infinite horizon.

x˙=Ax+bu,u=−Kx

A=01−25, b=03, Q=1002, r=5

J=0∞(xTQx+ru2)dt

# Simulink model

The plant is described in state space, the state feedback applies the gain K obtained from the algebraic Riccati equation, and the logging blocks record the states, control and accumulated cost. A screenshot of the model is shown below.

# Simulation results

The trajectories of the states x1, x2, the control u and the accumulated cost J are obtained for x(0) = [1, 0]. The closed-loop matrix A - bK is Hurwitz. The effect of the control weight r and of the matrix Q on the transient is studied, as well as the response to a 5% change of the gain K while stability is preserved; for each experiment x1, x2, u and J are plotted.

K = [0.1361, 3.4746]

real parts of the eigenvalues = [-4.9358, -0.4879]

steady-state cost J = x0' P x0 = 12.8122

# Comparison with Python / analytical reference

Numerical results of the Simulink model compared against the Python / analytical reference.

| Quantity | Simulink | Python reference |
| --- | --- | --- |
| K[0] | 0.1361 | 0.1361 |
| K[1] | 3.4746 | 3.4746 |
| J_inf | 12.8122 | 12.8122 |
| Re(eig) [1] | -4.9358 | -4.9358 |
| Re(eig) [2] | -0.4879 | -0.4879 |

# Conclusion

The controller gain K, the steady-state cost J and the closed-loop eigenvalues obtained in Simulink agree with the Python reference to better than 1e-3, which confirms the correctness of the model.