# Work 5. Optimal observer (Kalman filter) and LQG controller in Simulink (variant 5)

# Problem statement

A linear plant of variant 5 with process noise and measurement noise is considered. The goal is to design the optimal observer (Kalman filter) and, based on it, the LQG controller.

x˙=Ax+bu+Gw,y=Cx+v

A=0−41−4, b=87, C=[1 0], G=I

W=43.53.55, V=5

# Simulink model

The plant is driven by process noise, the measurement is noisy, and the observer reconstructs the state with the Kalman gain L = P C' V^-1. The LQG controller forms the control u = -K x_hat from the state estimate. The logging blocks record the estimates, the estimation errors and the control. A screenshot of the model is shown below.

# Simulation results

The trajectories of the state estimates, the estimation errors and the LQG controller signals are obtained. The process noise is formed as cholW*(unit white noise), so that the spectral density of the process noise equals the prescribed matrix W; the measurement noise has intensity V. The Kalman filter keeps the estimation error in a small neighbourhood of zero of order sqrt(tr P). The observer optimality cost J = M[e_H' e_H] (the expected squared estimation error) is computed; in steady state E[e e'] = P, hence J = tr(P). The error dynamics e' = (A-LC)e + Gw - Lv do not depend on the control, so J is the same for u = sin t and for the LQG feedback. The effect of a non-optimal gain L and of changing the noise intensities W and V on the estimation quality is studied. The gains L, K and the closed-loop LQG eigenvalues agree with the Python reference.

L = [0.2738, 0.0906]

K = [0.2046, 0.4465]

estimation-error cost J = tr(P) = 2.1024

real parts of the LQG eigenvalues = [-7.6683, -1.0942]

# Comparison with Python / analytical reference

Numerical results of the Simulink model compared against the Python / analytical reference.

| Quantity | Simulink | Python reference |
| --- | --- | --- |
| L[0] | 0.2738 | 0.2738 |
| L[1] | 0.0906 | 0.0906 |
| K[0] | 0.2046 | 0.2046 |
| K[1] | 0.4465 | 0.4465 |
| Re(eig LQG) [1] | -7.6683 | -7.6683 |
| Re(eig LQG) [2] | -1.0942 | -1.0942 |

# Conclusion

The random noise realisation in Simulink is not reproduced sample-by-sample with Python; the deterministic quantities L, K and the LQG eigenvalues are compared instead. These quantities agree with the Python reference to better than 1e-3, which confirms the correctness of the observer and LQG controller design.