# Work 3. Optimal control design for LTI plant (LQR)

# Problem statement

Linear plant and quadratic cost of variant 5:

x˙=Ax+bu,u=−Kx

A=01−25, b=03, Q=1002, r=5

J=0∞(xTQx+ru2)dt

# Optimal controller (Riccati), simulation of x1, x2, u, J

Solving the algebraic Riccati equation gives P and the gain K=r^-1 b'P. The closed-loop matrix A-bK is Hurwitz. The variables x1, x2, u and the accumulated cost J are plotted separately for x(0)=[1,0]'.

K = ['0.1361', '3.4746']

closed-loop eigenvalue real parts = ['-0.4879', '-4.9358']

steady-state cost J = x0' P x0 = 12.8122

accumulated cost over the simulation (T=8) = 12.8054

# Slightly changed K (stability preserved)

Increasing the gain by 5% keeps stability but raises the steady-state cost, confirming K is optimal. Experiment 2 is repeated with the same simulation time and x1, x2, u, J are compared with the optimal case.

perturbed closed-loop eig real parts = ['-0.4413', '-5.5036']

optimal J = 12.8054, perturbed J = 12.8237

# Three r and three Q=kQ*

For three values of r and three matrices Q=kQ* the variables x1, x2, u and J are plotted. Larger r penalizes control effort (slower, cheaper control); larger Q penalizes the state (faster response).