# Work 4. Optimal control design (Bellman's dynamic programming)

# Problem statement

Finite-horizon problem; A,b,Q,r are taken from Work 2:

x˙=Ax+bu,x(0)=[1, 0]T

J=05(xTQx+ru2)dt

A=01−6−5, b=01, Q=diag(1,4), r=5

# Synthesis (Bellman / differential Riccati)

Dynamic programming yields the differential Riccati equation integrated backward from P(T)=0; the time-varying gain is K(t)=r^-1 b'P(t). The variables x1, x2, u and the accumulated cost J are plotted separately.

−P˙=ATP+PA−Pbr−1bTP+Q,P(5)=0

u(t)=−r−1bTP(t)x(t)

K(0) = ['0.0166', '0.0826']

optimal cost J = 2.9023

# Cost for changed controller parameters

Scaling the optimal gain up or down increases the cost, confirming the dynamic-programming solution is optimal.