# Work 5. Optimal observer (Kalman filter) and LQG controller

# Problem statement

Plant with process and measurement noise, variant 5:

x˙=Ax+bu+Gw,y=Cx+v

A=0−41−4, b=87, C=[1 0], G=I

W=43.53.55, V=5

# Optimal observer (Kalman gain)

The filter Riccati equation gives P and the gain L=PC'V^-1. The estimation error decays to a small steady state.

AP+PAT+GWGT−PCTV−1CP=0

L=PCTV−1

L = ['0.2738', '0.0906']

# Changed L (stability preserved)

The observer gain L is slightly changed while the error dynamics stay stable, and experiment 2 is repeated with the same simulation time. A non-optimal L degrades the estimation error.

# Changed W (symmetric, positive definite)

W is replaced by W=[[8,3.5],[3.5,10]], which is symmetric and positive definite, and experiment 2 is repeated. Larger process noise W raises the gain and the steady-state estimation error.

# Changed V (positive)

V is replaced by V=1 (positive), and experiment 2 is repeated. Smaller measurement noise V makes the filter trust the measurements more, which reduces the estimation error.

# LQG controller

Combining the Kalman estimate with the LQR gain (Q,r from Work 2) gives the LQG controller u=-K x_hat; the closed loop is stable.

K = ['0.2046', '0.4465']

LQG closed-loop eig real parts = ['-1.0942', '-7.6683']